AICELLE  Display

is sense and simplicity

SINCE 1993

龙凤芯，自然光，更健康

AICELLE Display Systems,

Natural Light for Healthy Life

powered air-purifying respirator

FA400

Merak A天璇

Innovative  power

comprehensive  protection

★ EN   12941:1998+A1:2003+A2:2008  TH2   PR  SL

★ Effective protection against

breathing,eyes and skin.

★ Designed to enhance comfort.

require a fit test,which is more

convenient and effective.

★ The loose-fit head mask does not

Ultra clear large window wider  horizons

★ Easy to operate,comfortable and lightweight.

★ Various ADF options are available.

★Moderate strength shell,high

toughness  and  strong  impact  resistance.

★ All-round eye and face welding  protection.

★ UV/IR protection class DIN16.

Auto-darkening welding Helmet

C

富 相

Phecda A 天玑

ADF

ADF0001                                            ADF0002

Technical parameter

| Model | ADF0001 | ADF0002 |
| --- | --- | --- |
| Viewing Area | 98*87mm | 98*89mm |
| CE Optical Class | 1/1/1/2 | 1/1/1/2 |
| Arc Sensors | 4PCS | 4PCS |
| Sensitivity | Stepless Variable(Internal) | 1-6 levels digital control |
| Switch Time(23℃) | <1/30000S | <1/30000S |
| Delay Time | 0.1-1.0s Stepless Variable | 0.1-1.0s digital control |
| Power Supply | Solar Cell,1*CR2450 Li-Battery Change Required | Solar Cell,1*CR2450 Li-Battery Change Required |
| Shade Control | Stepless Variable(Internal) | Internal(digital) |
| Shade State | DIN4/5-8/9-13 | DIN4/5-8/9-13 |
| Operating Temp | -10℃~+60℃ | -10℃~+60℃ |

一、Dust filter box SE0003,Mars net+HEPA.

● Conforms to EN12941 TH2P standard.

●  Filtration  efficiency>99.98%.

● The spark catcher screen reduces the

contact of sparks and other hot materials

with the filter cotton.

二、Composite  filter  cartridge  SE0009,    Mars   net+activated   carbon+HEPA.

● Conforms to EN 12941 TH2A1P standard.

●  Protection against dust,

filtration   efficiency>99.98%.

●  Odor  removal,smoke  odor.

●Protection  against  organic  gases/vapors.

Thread  connection:EN148  thread440mmX1/7"

Lithium Battery

一 、Capacity  2600mAh  standard  battery

● Maximum range of 10 hours

(New filter cartridge and fully charged state).

● Lithium batteries are charged without memory effect. Charging status can be easily pulled out.

二、Capacity  3000mAh   high  capacity   battery

● Maximum range of 12 hours

(New filter cartridge and fully charged state).

● Lithium batteries are charged without memory effect. Charging status can be easily pulled out.

ON      OFF                           mn     点 r

N

ON

Tap  to  power  on,tap  to  select  after  power  on  High  air flow or low air flow.

OFF

Press  and  hold  for2  seconds  to  turn  off  the  machine.

Y     Low-speed   air   flow

LOW  Provides  a  steady  airflow  of  180L/min.

High-grade air flow

HIGH  Provides  a  stable  airflow  of  220L/min.

Low airflow alarm

AIR

FLOW

When the  airflow  is  low,the AlR  FLOW  light  of the  machine will  light  up,sounding  an   alarm  and   accompanied  by

vibration,the   user   should   leave   the   workplace   immediately and go to a safe area to check the machine.

口 Low battery alarm

LOWWhen  the  battery  power  is  low,the  machine  LOWBATTERY BATTERY light comes on and An alarm sounds and vibrates.

The  user  should  leave  the  workplace  immediately  and go to a safe area to  replace or recharge the  battery.

Filter cartridge  replacement

CHANGE When the filter cartridge  is severely clogged and the

FILTER  machine's  CHANGE  FILTER  light  comes  on,the  user should

replace the cartridge with a  new one as soon as  possible.

十

临

富 相

Dubhe A天枢

十

Wide  range of applications

With  respirator  as  the  core,by  equipping  with  welding  cap and  hood  to  create  a  variety  of  combination  solutions  to     meet  the  needs  of  various  industries.

201mm

247mm                                      105mm

J

the  latch  down.

深圳市龙凤芯科技有限公司

地址：深圳市宝安区西乡铁仔路凤凰智谷A509

服务电话：4001-897-687

官网：http://www.aicelle.com

http://www.aicelledisplay.com

② Remove the battery.

①Push